Lab Assignment 3

- Perform the following operation on a given Boolean expression:

F(ABC) = ABC + A (B’ + C’)

  - Write a truth table for each Boolean expression.

  - Draw a schematic diagram for each Boolean expression.

  - Write a Verilog code for each Boolean expression and then test using wave form and compare with truth table whether your circuit produced same output or not?

Design bench.sv

module gate1 (input a,b,c, output y);

assign y = ((a&b&c)|a&(~b|~c));

endmodule

Test bench.sv

module tb_gate1();

reg A,B,C;

wire Y;

//print

//Ayushman Pranav E21cseu0245

gate1 a1(.a(A),.b(B),.c(C),.y(Y));

initial begin

A=1; B=1; C=1; #5;

$display("gate1");

$display("A B C Result");

$display("%b %b %b   %b",A,B,C,Y);

A=1; B=1; C=0; #5;

$display("%b %b %b   %b",A,B,C,Y);

A=1; B=0; C=1; #5;

$display("%b %b %b   %b",A,B,C,Y);

A=1; B=0; C=0; #5;

$display("%b %b %b   %b",A,B,C,Y);

A=0; B=1; C=1;#5;

$display("%b %b %b   %b",A,B,C,Y);

A=0; B=1; C=0;#5;

$display("%b %b %b   %b",A,B,C,Y);

A=0; B=0; C=1;#5;

$display("%b %b %b   %b",A,B,C,Y);

A=0; B=0; C=0;#5;

$display("%b %b %b   %b",A,B,C,Y);

end

// EP wave

initial begin

$dumpfile("dump.vcd");

$dumpvars(1);

end

endmodule

- Perform the following operation on a given Boolean expression:

F(ABCD) = A’B’C’D’ + A’BCD’ + A’B’C’

  - Write a truth table for each Boolean expression.

  - Draw a schematic diagram for each Boolean expression.

  - Write a Verilog code for each Boolean expression and then test using wave form and compare with truth table whether your circuit produced same output or not?

Design bench.sv

module gate1 (input a,b,c,d, output y);

assign y = ((~a&~b&~c&~d)|(~a&b&c&~d)|(~a&~b&~c));

endmodule

Test bench.sv

module tb_gate();

reg A,B,C,D;

wire Y;

//print

//Ayushman Pranav E21cseu0245

gate1 a1(.a(A),.b(B),.c(C),.d(D),.y(Y));

initial begin

A=1;B=1;C=1;D=1;#5;

$display("A B C D Result");

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=1;B=1;C=1;D=0;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=1;B=1;C=0;D=1;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=1;B=1;C=0;D=0;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=1;B=0;C=1;D=1;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=1;B=0;C=1;D=0;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=1;B=0;C=0;D=1;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=1;B=0;C=0;D=0;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=0;B=1;C=1;D=1;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=0;B=1;C=1;D=0;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=0;B=1;C=0;D=1;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=0;B=1;C=0;D=0;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=0;B=0;C=1;D=1;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=0;B=0;C=1;D=0;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=0;B=0;C=0;D=1;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

A=0;B=0;C=0;D=0;#5;

$display("%b %b %b %b   %b",A,B,C,D,Y);

end

// EP wave

initial begin

$dumpfile("dump.vcd");

$dumpvars(1);

end

endmodule

- Representation of XOR gate using only NAND gates and perform the following operations:

  - Derive the Boolean expression.

  - Write the truth table for the above expression

  - Write a Verilog code for each Boolean expression and then test using wave form and compare with truth table whether your circuit produced same output or not?

Design bench.sv

module

gate1 (input a,b, output y);

assign y = ~(~(a&(~(a&b)))&~(b&~(a&b)));

endmodule

Test bench.sv

module tb_gate();

reg A,B;

wire Y;

//print

//Ayushman Pranav E21cseu0245

gate1 a1(.a(A),.b(B),.y(Y));

initial begin

A=1;B=1;#5;

$display("A B Result");

$display("%b %b   %b",A,B,Y);

A=1;B=0;#5;

$display("%b %b   %b",A,B,Y);

A=0;B=1;#5;

$display("%b %b   %b",A,B,Y);

A=0;B=0;#5;

$display("%b %b   %b",A,B,Y);

end

// EP wave

initial begin

$dumpfile("dump.vcd");

$dumpvars(1);

end

endmodule

- For the given circuit diagram do the following:

- Derive the Boolean expression.

- Write the truth table for the above expression

- Write a Verilog code for each Boolean expression and then test using wave form and compare with truth table whether your circuit produced same output or not?

Design bench.sv

module

gate1 (input a,b, output y);

assign y = ~(~(~(a&(~(a&b)))&~(b&~(a&b))));

endmodule

Test bench.sv

module tb_gate;

reg A,B;

wire Y;

//print

//Ayushman Pranav E21CSEU0245

gate1 a1(.a(A),.b(B),.y(Y));

initial

begin

A=1; B=1; #5;

$display("%b %b   %b",A,B,Y);

A=1; B=0; #5;

$display("%b %b   %b",A,B,Y);

A=0; B=1; #5;

$display("%b %b   %b",A,B,Y);

A=0; B=0; #5;

$display("%b %b   %b",A,B,Y);

end

initial begin

$dumpfile("dump.vcd");

$dumpvars(1);

end

endmodule